Program 5X86.EXE

Author Peter N Moss

E-Mail Address pmoss@yoda.alt.za

Version B6  16 December 1995.

Source code: Source code is not included.

Compiled with Turbo Pascal V5.5 and TASM V1.1

Files:

5X86xx.ZIP      The combined zipped distribution package

5X86.EXE        The program file

5X86.REG        Registration form

5X86.DOC        This file

The program:

The program detects the presence of a 5x86 processor and will not

run if a Cyrix/IBM 5x86 is not found.

5x86.EXE will report and enable the modification of the performance

control register of the Cyrix 5x86 (TM) processor.  Access to register

bits marked reserved (*) is not supported other than to report status

if in some cases non zero.

Useage: 5x86 <[/H|?] [/Q]> </BTB_EN=[on|off] /LOOP_EN=[on|off]

/LSSER=[on|off] /RSTK_EN=[on|off] /BWRT=[on|off]

/WT1=[on|off]  /LINBRST=[on|off] /MEM_BYP=[on|off]

/DTE_EN=[on|off] /FP_FAST=[on|off]> <[/ICD|/CWT|/CWB]> <[/T]>

Where BTB_EN  Branch prediction

LOOP_EN Loop (Prefetch buffer)

LSSER   Load store serialise, reorder

RSTK_EN Return stack

BWRT    Burst write cycle 4 dwords on cache line replacement.

WT1     Write through region one (640kb..1Mb).

LINBRST Linear burst mode.

MEM_BYP Memory read bypassing

DTE_EN  Directory table cache

FP_FAST Fast NPU exception reporting

ICD     CPU Internal cache disable

CWT     CPU cache enable, write through mode

CWB     CPU cache enable, write back mode

Q = Program version

H,? = Help

T = Status of PCR0 register

E = (Reserved) Debug report.  Status of PCR0, CCR1..4

and CR0

Note that T may be combined with register bit settings

and will report status after the register has been set

by reading the register. Thus giving confirmation that

the register has been set correctly ie 5x86 /LSSER=on

/BTB_EN=on /RTS=off /t

E is included for debugging and expansion.  The report

is more than 25 lines so it would be advisable to use

this with either a DOS pipe to more (| more) or

redirection to a file (> filename).  It is is not part

of the program and is not expected to continue beyound

its current use.

On = enable, 1; Off = disable, 0.  On and off are valid and

all other values are treated as a error.

The first three characters only need be entered. LIN=on is the

same as LINBRST=on.  Less than three characters of the register

bit name will cause an error.

The cache enable/disable parameters are only valid for the CPU

at level 0.  DOS operates at level 0 (ring 0).

Upper case or lower case characters may be used.  The order of

register input is not important.  H, ? or Q will terminate any

other command line parameters.  T may be placed anywhere on the

command line.

Command line input may use any legal DOS seperator, space tab /

; ect.

Errorlevel:

5 = No Cyrix 5x86.

1 = Command line entry error

0 = Normal termination.

Register: Performance control register 0

The following bits of this register perform various functions detailed

with the register bit name.

Register PRC0 Index 20h

Bit map:

----------------------------------------------------------------

7      6      5      4      3       2         1        0

----------------------------------------------------------------

|LSSER|  Res4|  Res3|  Res2|  Res1|  LOOP_EN|  BTB_EN|  RSTK_EN|

----------------------------------------------------------------

Bit    Name     Function

7      LSSER    1 = Load store serialise enable (Reorder disable)

6      Res4     BTB Test register

5      Res3     Reserved

4      Res2     Reordering of locked misalighned loads

3      Res1     All instructions stalled to serialise pipeline

2      LOOP_EN  Loop enable

1      BTB_EN   Branch target buffer enable

0      RSTK_EN  Return stack enable

Access to this register is via I/O ports 22h (Index) and 23h (Data)

and the MAP_EN bits of the CCR3 register.  Full details of the register

address and access requirements can be found in the Cyrix 5x86 (TM)

processor data book #94192-00.

RSTK_EN: 1 = The return stack is enabled and RET instructions will

speculatively execute the code following the associated CALL

to improve performance.

BTB_EN:  1 = Branch target buffer enable.  Branch prediction occurs.

LOOP_EN: 1 = The CPU will not flush the prefetch buffer if the

destination jump is already in the prefetch buffer

eliminating the need for a read from the cache.

LSSER:   1 = All memory reads and writes will occur in execution

order. Reordering disabled.

LSSER should be set to ensure that memory mapped I/O devices

operating outside of the address range 640k..1M will operate

correctly.

Memory accesses in the range 640k..1M will always be issued

in execution order.

Usage and optimisation:

In setting the CPU please note the the results are very dependent on

the correct setup of the CMOS parameters.  Particular attention should

be placed on the L2 cache settings as well as memory read and write

wait states.  It is suggested that these setting be optimised first

before attempting to "tune" the CPU performance control register.

There maybe some interaction of the CMOS settings and CPU settings

requiring iteration to find the most suitable.

The Cyrix 5x86 as other Cyrix processors requires that the motherboard

and BIOS support the on chip cache (L1) as well as other signal pins.

For this reason the motherboard jumpers and CMOS must be set correctly.

The cache is enabled/disabled by setting CPU CR0 register which can

only be accessed when the CPU is operating at level 0 or ring 0.

This feature is not to be used in protected mode (32 bit programs) such

as Windows, Unix ect.  No checking is done at present allthough

possible.  The user is expected to read and understand this

statement and not foolishly attempt to modify the cache setting when

operating the CPU at any other level than 0.  The use of this feature

is only included for those motherboards without Cyrix cache support.

Some problems have been noted with LSSER set to 0 the default when

other functions (bits) have been enabled.

Suggested BIOS default values are:

CCR2  BWRT    = 0

CCR4  MEM_BYP = 1

DTE_EN  = 1

FP_FAST = 0

IORT    = 0

PCR0  RSTACK  = 0

BTB     = 0

LOOP    = 0

LSSER   = 1 for PCI memory mapped devices

LSSER   = 0 for systems without PCI

The parameters for use can be found by entering h or ? on the command

line.  When the optimum parameters have been found path\5x86 {parameters}

may be placed in your autoexec.bat file with those parameters so that

the PC is set on startup.

What are we dealing with?

Memory management:

Load-store reordering; that prioritises memory reads required by the

integer unit over writes to external memory.

Memory-read bypassing; that eliminates unnecessary memory reads by using

valid data still in the execution unit.

Branch prediction:

The branch prediction target buffer (BTB) is used to store branch target

addresses and branch prediction information.

The target address of a RET instruction is dynanic rather than static

and return addresses are cached in a return stack rather than the BTB.

The return address is pushed on the return stack during a CALL

instruction and poped during the corresponding RET instruction.

Functionality, type and rights:

5x86.EXE is not crippled or contain nagware.  It is shareware, all

rights are reserved by Peter N Moss.

Copyright:

5x86.EXE and 5x86.DOC are copyright 1995 Peter N Moss.

Disclaimer:

No claim is made for the fitness of purpose of this software and no

claim for damages of any nature will be accepted.  The program is

for use at entirely your own risk.

Distribution:

You may distribute the files only on condition that they are not

altered and this document file is included.  The latest version can

be found at ftp://compnt.cs.unp.ac.za/uploads/cyrix/5x86(Ver).zip.

Updates to registered users will only be sent via Internet e-mail

without charge.  Disk updates will be charged at the cost of

materials and postage.

Reports and support:

Whilst reports of results or bugs are welcomed, it is not possible to

support questions on how to setup or optimise any motherboards.  You

are referred to the dealer that sold the board and or CPU.  Registered

users can expect free udate notification and as much help as can be

given.

Thanks:  Mr C Chan, Mr K J Wren and Mr J Stiel for testing and

feedback.

Trademarks:  Cyrix is a registered trademark of Cyrix Corp.

Cx5x86 is a registered trademark of Cyrix Corp.

Payment: Payment of US$5.00 or the equivelent is such a small amount

that anyone that uses the program continuously should contribute to

the development of it.  It will also insure that the program continues

to be developed.  You are offered 30 days free trial for evaluation

only.  After that period has expired and registration or payment has

not taken place then the program must not be used.

For money order payment from outside of South Africa please add $4.00.

Payment in cash, cheques or money order may be made to:

P N Moss

P O Box 212144

Oribi

Pietermaritzburg

Rep of South Africa, 3205

When making payment please include your e-mail address if you have one

and your return mail address.

Registration form:  5X86.REG